Keele University Carbon Emissions Annual Report:

01 August 2024- 31 July 2025

1.	INTRODUCTION	1

2.	REPORTED CARBON EMISSIONS AND RELATION TO TARGETS	2

1.1	Scope 1 and 2 emissions	2

1.2	Scope 3 emissions	5

3.	REVIEW OF PROGRESS AND DIRECTION OF TRAVEL	7

4.	OUTLOOK AND REDUCTION TARGETS FOR NEXT REPORTING YEAR	7

5.	CURRENT STATUS AND OUTLOOK FOR CARBON COMPENSATION.	9

## INTRODUCTION

This report presents a summary of Keele University’s carbon emissions over the annual period 01 August 2024 to 31st July 2025, with a comparison against an original baseline established in 2005, updated baseline for the year 2018-2019, and 2030 targets. It follows information and guidance within the Carbon Accounting, Reporting and Management Code of Practice 2024-2029 which can be found within Keele University’s policy zone.

Table 1 summarises the buildings/space for which emissions associated with energy consumption are included in Scope 1 and 2 accounting, and those for which they are categorised as Scope 3. For consistency and transparency, it is primarily in agreement with the data reported through the Higher Education Statistics Agency Estates Management Record (HESA EMR) in relation to Scope 1 and 2 emissions and Scope 3 emissions relating to supply chain, waste, water and commuting, with an explanation provided where this is not the case. As the HESA EMR does not encompass all greenhouse gas emission sources required for a comprehensive organisational inventory, additional categories have been accounted for in accordance with the Greenhouse Gas Protocol using the EAUC Standardised Carbon Emissions Reporting Framework (SCERF) for Further and Higher Education. This approach supports a more complete and robust assessment of the University's carbon footprint while maintaining comparability with sector reporting requirements.

Table 1. Buildings included in Keele University Scope 1&2 and Scope 3-leased assets (Full list in Appendix)

| Assets in boundary for Scope 1 &2 | GIA (m2) | Assets in boundary for Scope 3 | GIA (m2) |
| --- | --- | --- | --- |
| University-owned student accommodation | 62,654 | Staff housing (University owned) |  |
| University-owned and occupied non-residential buildings/ space | 123,701 | Buildings/space rented by University with no or limited operational control |  |
| University-owned buildings with mix of University use and leased space based on extent of operational control/ proportion of space |  | Buildings/space leased by University with no or limited operational control where energy consumption data is available |  |
| Total GIA (m2) | 186,354 |  | 15,216 |

UK Government GHG Conversion Factors for Company Reporting are published in June/July each year by the Department for Energy Security & Net Zero (DESNZ) and Department for Environment Food & Rural Affairs (DEFRA). Table 2 displays the equivalent carbon per kilowatt hour of gas and electricity consumption for the reporting years 2018-2025, showing a marked decrease in electricity conversion factor for 2025 reporting. This reflects the increasing proportion of renewable energy sources within the UK grid electricity mix since 2023.

Table 2. UK Government GHG Conversion Factors for Company Reporting

|  | kgCO2e/kWh |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reporting Year | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 |
| Natural Gas (Gross CV) | 0.1839 | 0.1839 | 0.1832 | 0.18 | 0.18 | 0.1829 | 0.1830 |
| Grid Electricity | 0.2556 | 0.2331 | 0.2123 | 0.1934 | 0.2071 | 0.2071 | 0.1770 |

## REPORTED CARBON EMISSIONS AND RELATION TO TARGETS

### Scope 1 and 2 emissions

Table 3 displays a breakdown and total of Scope 1 and 2 emissions for 2024-25 with a description of source data and level of calculation methodology as defined by the SCERF, and comparison with baseline year (2018-19) and previous year’s emissions.

Table 3. 2022-23 Scope 1 and 2 category emissions compared with baseline year emissions

| GHG emission Scope/category | Keele University inventory | Baseline year 2018/19 emissions (tCO2e) | 2023/24 emissions (previous year) (tCO2e) | 2024/25 (reporting year) emissions (tCO2e) | SCERF Level of Reporting | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Scope 1- Natural gas | Emissions from combustion of natural gas in on-site boilers applicable to buildings/space as defined within Table 1,and in teaching/research laboratories | 4677 | 4745 | 4792 | Advanced | Based on metered consumption/ validated billing data for assets in boundary of Scope 1 & 2 |
| Scope 1- Fleet (owned/operated) | Emissions from fuel used in University-owned or leased vehicles | 28 | 44 | 45 | Advanced |  |
| Scope 1- Refrigerants, research- based f-gas, volatile organic compounds | Emissions from leakage of refrigerants from air-conditioning applicable to buildings/space as defined within Table 2 | 34 | 77 | 77 | Advanced | Estimation from assuming 5% leakage rates and top-up/discharge data. Refrigerant data is not included in HESA EMR. |
| Scope 1- Other fuels | Emissions from combustion of fuel in generators applicable to buildings/space as defined within Table 2 | Not reported. | Not reported. | Not reported | N/A | Judged as immaterial (0.01-0.05% of Scope 1 and 2 emissions) |
| Scope 2- Indirect GHG emissions from electricity consumption | Emissions from purchased electricity for buildings/space as defined within Table 2 | 2776 | 1588 | 1372 | Advanced | Based on metered consumption/ validated billing data for assets in boundary of Scope 1 & 2 |
| TOTAL SCOPE 1 & 2 EMISSIONS |  | 7515 | 6454 | 6286 |  | Minimum 2030 target= 3717 tCO2e Aspirational 2030 target=752 tCO2e |

As shown in Table 3, total Scope 1 and 2 emissions for 2024-25 year were estimated as 6286 tCO2e and are shown in Figure 1 in relation to baseline and target emissions. It represents a 54 % reduction compared with an original baseline year of 2005 and 16 % reduction compared with an updated 2018-19 baseline emissions. This falls short of the targeted 28 % reduction from baseline year to be on track to meet the 2030 Science Based Target Initiative (SBTi) 1.5 oC aligned near-term (minimum) reduction target, and the required 49 % reduction to be on track to meet the 2030 SBTi 1.5oC-aligned net zero (aspirational) target.

Figure 1. 2024-25 Scope 1 and 2 emissions in relation to previous years, original 2005 baseline, updated 2018-19 baseline year and 2030 target

### Scope 3 emissions

Table 4 displays a breakdown and total of Scope 3 emissions for 2024-25 with a description of source data and level of calculation methodology as defined by the SCERF, and comparison with baseline year (2018-19) emissions.

Table 4. Scope 3 category emissions compared with baseline and previous year emissions, with description of calculation methodology and additional notes on boundaries/measurement

| GHG emissions Scope 3 category | Keele University inventory | Baseline year 2018/19 emissions (tCO2e) | Previous year 2023/24 emissions (tCO2e) | Reporting Year 2024/25 emissions (tCO2e) | SCERF Level of Reporting | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Procurement and supply chain | Emissions associated with procurement & supply chain of goods and services purchased for the operation of the University. | 21,051 | 34,444 | 18,763 | Basic | ‘Scope 3’ report provided by NWUPC based on KU purchasing ledger |
| Capital Goods | Emissions associated with purchase of tangible assets | (included in above) | (included in above) | (included in above) | Basic |  |
| Fuel and energy related activities | Upstream emissions associated with fuels and energy in Scope 1 & 2 (i.e., emissions associated with getting fuel/energy to point of use including well-to- tank and transmission & distribution) | 1047* | 1442 | 1433 | Advanced | *Baseline figure of 270 stated in ‘Carbon Accounting, Reporting and Management Code of Practice 2024-2029’ revised due to accounting error |
| Water supply | Emissions associated with water supply | 111 | 20 | 24 | Advanced | Based on metered water consumption validated billing data |
| Waste generated in operations | Emissions associated with waste disposal and treatment of waste, recycling and wastewater. | 254 | 40 | 26 | Advanced | Based on solid waste and wastewater validated billing data |
| Business Travel | Emissions associated with travel and accommodation of staff for business-related activities. | 1354 | 2433 | 1979 | Advanced |  |
| Employee Commuting | Emissions from staff commuting as well as emissions associated with remote working. | 9189 (Student commuting accounted for in Employee Commuting) | 2326 | 2326 | Basic | Remote working emissions not accounted for |
| Downstream transportation and distribution | Emissions from student travel, including daily commuting and travel between home address and University for UK and international students. |  | 5001 | 5001 | Basic | International student travel not accounted for |
| Upstream leased assets | Emissions from rented buildings and vehicles | N/A | N/A | N/A |  |  |
| Downstream leased assets | Emissions from leased buildings. | 178 (leased staff housing only) | 1290 | 991 | Advanced | As defined in Table 1 |
| Investments | Investments, endowments | Not measured | Not measured | Not measured |  |  |
| TOTAL SCOPE 3 EMISSIONS |  | 33,184 | 46,996 | 30,543 |  | Minimum 2030 target= 17,375 |

As shown in Table 4, total Scope 3 emissions for 2024-25 year were estimated as 30,543 tCO2e and are shown in Figure 2 in relation to baseline, previous year and target emissions. It represents a 8 % decrease compared with 2018-19 baseline emissions, which compares with a targeted 25 % reduction to be on track to meet the 2030 minimum reduction target. However, it should be noted that the immaturity of measurement method for supply chain emissions, commuting emissions, and omission of student international travel emissions and investments & pensions, means estimates and targets are likely to change significantly as Keele University works towards increased accuracy and completeness of accounting along with the wider HE sector. In particular, the dominance of supply chain emissions, challenge of quantification, and widely used methodology of spend-based calculation based on DEFRA conversion factors, means reported Scope 3 emissions can fluctuate widely from year to year. For example, the increase for previous year (23/24) emissions was mainly due to upgrades to ICT equipment across the University, and does not reflect Keele’s efforts to work with its main supplier to create a more circular economy, which hopefully can be captured in future reporting.

Figure 2. 2024-25 Scope 3 emissions in relation to previous year, 2018-19 baseline year and 2030 target

## REVIEW OF PROGRESS AND DIRECTION OF TRAVEL

As stated above, combined Scope 1 and 2 emissions have reduced by 16 % from 2018-19 baseline level. The reduction is due to the significant decrease in grid electricity consumption of ~33% due to the construction and commissioning of the Low Carbon Energy Generation Park (LCEGP) in 2021/22.  Together with a reduction in grid electricity GHG conversion factor (Table 1), this resulted in a 51 % reduction in Scope 2 emissions. However natural gas consumption and (owing to the consistent GHG conversion factor of natural gas) associated Scope 1 emissions increased by 2 %, despite a switch-off of the Combined Heat and Power (CHP) plant in 2021 and installation of electric boilers (which auto-switch from gas boilers when excess renewable energy is generated from the LCEG Park). This small increase needs to be considered in the context of a 6 % increase in gross internal area (GIA) of the estate and 12 % increase in total full-time equivalent (FTE) staff and students for 24-25 compared with 2018-19. Nonetheless, it highlights the need to target reduction in campus heat demand and efficiency of supply as well as the longer-term aim to supply virtually all energy demand by non-fossil fuel-based systems.

## OUTLOOK AND REDUCTION TARGETS FOR NEXT REPORTING YEAR

While the University remains committed to achieving its 2030 Science Based Targets, the 2024-25 reporting year highlights the need for a pragmatic and achievable trajectory over the next twelve months. Significant reductions in Scope 2 emissions have already been realised through the Low Carbon Energy Generation Park and the continuing decarbonisation of the UK electricity grid. Further reductions in Scope 1 emissions are expected to be more challenging and will require targeted interventions to reduce heat demand, improve building performance, optimise heating controls and progress the transition away from fossil fuel-based heating systems. Scope 3 emissions continue to represent the largest proportion of Keele's carbon footprint and are subject to greater uncertainty due to evolving methodologies and data quality, particularly for procurement and supply chain emissions. Consequently, 2025-26 targets have been set to reflect areas where the University has a reasonable degree of influence and where year-on-year reductions are considered credible and achievable.

The proposed targets displayed in Table 5 represent an overall reduction of approximately 6 % in total reported emissions compared with the 2024-25 reporting year. This would reduce total emissions from 36,829 tCO₂e to approximately 34,800 tCO₂e. Scope 1 and 2 emissions are targeted to reduce by approximately 8 %, while Scope 3 emissions are targeted to reduce by approximately 5%, recognising the greater uncertainty and external influences associated with indirect emissions categories.

Table 5. Proposed emission reduction targets for 2025-26

| Emission Category | 2024-25 Actual (tCO₂e) | 2025-26 Target (tCO₂e) | % Reduction |
| --- | --- | --- | --- |
| Natural Gas | 4,792 | 4,500 | 6 % |
| Fleet | 45 | 40 | 11 % |
| Refrigerants & VOCs | 77 | 77 |  |
| Purchased Electricity | 1,372 | 1,200 | 13 % |
| Total Scope 1 & 2 | 6,286 | 5,807 | 8 % |
| Procurement & Supply Chain (including Capital Goods) | 18,763 | 17,825 | 5 % |
| Fuel & Energy Related Activities | 1,433 | 1,350 | 6 % |
| Water Supply | 24 | 20 | 17 % |
| Waste Generated in Operations | 26 | 20 | 23 % |
| Business Travel | 1,979 | 1,850 | 7 % |
| Staff Commuting & Working from Home | 2,326 | 2,200 | 5 % |
| Student Travel | 5,001 | 4,850 | 3 % |
| Downstream Leased Assets | 991 | 900 | 9 % |
| Total Scope 3 | 30,543 | 29,015 | 5 % |
| Total Scope 1, 2 & 3 | 36,829 | 34,815 | 6 % |

It is recognised that the proposed 2025-26 reduction targets fall short of the rate of emissions reduction required to achieve the University's 2030 targets. However, they have been developed to represent a credible and achievable level of progress over the next reporting period, taking account of current technological, financial and operational constraints, as well as uncertainties associated with several Scope 3 emissions categories. During 2026-27, the University will review its carbon reduction trajectory and targets in light of the findings of the emerging decarbonisation roadmap, improvements in emissions data quality, and progress on planned interventions, with the aim of establishing a pathway that appropriately balances ambition with deliverability.

## CURRENT STATUS AND OUTLOOK FOR CARBON COMPENSATION.

Carbon compensation remains a necessary but complex and evolving component of a net-zero pathway. The principle of prioritising deep emissions reductions before using carbon credits is widely accepted, and the voluntary carbon market continues to face challenges due to inconsistent quality, limited regulation, and concerns about the credibility and impacts of some projects. Recent initiatives, including the Integrity Council for the Voluntary Carbon Market’s Core Carbon Principles and the VCMI Claims Code of Practice, are helping to establish stronger standards and improve market transparency.

Despite these improvements, criticisms and uncertainty persist, highlighting the need for a cautious, integrity-focused approach. Keele therefore intends to develop a compensation strategy aligned with emerging best practice, including the Oxford Offsetting Principles and high-integrity procurement routes such as the EAUC Carbon Coalition, and exploring a ‘leading’ approach of setting an internal carbon price (ICP) based on the true socioenvironmental cost of emitting a tonne of equivalent carbon emissions. This ICP could be used to establish a fund to achieve Beyond Value Chain Mitigation (BVCM), by investing in projects that support climate mitigation, especially those that generate additional co-benefits for people and nature. A fuller explanation of this and Keele’s overall approach is provided within the Carbon Accounting, Reporting and Management Code of Practice .

It should be noted that the conversion factors for reporting emissions associated with UK grid electricity consumption lag actual carbon intensity of UK grid electricity by approximately 2 years.

As above

HESA Scope 1 and 2 emissions were reported as 6661 tCO2e due to exclusion of refrigerant-based emissions in EMR submission requirements

As required by the Science Based Target Initiative (SBTi)